A Co-Simulation Framework for Building Energy Management as a Testbed for Energy-Aware Data Movement Analysis

CHOMPHUNUCH WONGPHONG
Thammasat University, Thailand, chomphunuch.won@dome.tu.ac.th
PATCHARARAT WONGTA
Thammasat University, Thailand, patchararat.won@dome.tu.ac.th
ASST. PROF. DR. WORAWAN DIAZ CARBALLO
Thammasat University, Thailand, papong@tu.ac.th

ABSTRACT

Buildings consume more than one-third of global energy, with HVAC systems accounting for over 70%. Unpredictable occupant behavior causes energy variability of 100-300%, even in similar buildings. Co-simulation frameworks combining building physics (EnergyPlus) with agent-based occupant behavior models (Mesa) improve prediction accuracy but require HPC resources. However, frequent data exchange creates bottlenecks, increasing processing time by 20-50% and consuming 0.06-0.2 kWh/GB for network transfers. This paper presents Twin-B, a co-simulation testbed deployed on the LANTA supercomputer to profile energy-aware data movement in building simulation. Profiling 1,875 agents across 73 zones over three days reveals that cudaStreamSynchronize dominates 66.3% of CUDA API time due to 6,289 small (37.5-byte average) host-to-device transfers—a direct consequence of per-agent decision-making. These transfers waste 99.995% of PCIe bandwidth and consume 7.42 J energy. We demonstrate that batching agent operations could reduce energy consumption by 99.5% while achieving 1,200× speedup. This research establishes quantitative links between building simulation patterns and HPC efficiency, providing optimization strategies for sustainable digital twin systems.

Additional Keywords and Phrases:

Energy Efficiency, Co-simulation, Building Energy, Agent-based Model, HPC, GPU Profiling

# 1. INTRODUCTION

Energy efficiency in buildings, enhanced through co-simulation frameworks, offers a critical pathway toward achieving a 35% reduction in energy consumption per square meter by 2030. Studies identify six key factors influencing building energy use: occupant behavior, density, temperature, structure, system efficiency, and management policies. Of these, occupant behavior is the most significant yet unpredictable factor [2, 3]. While early research confirmed that simulating only physical factors is inadequate [4], computational constraints traditionally forced compromises in accuracy.

Advances in high-performance computing (HPC), particularly systems beyond Exascale with integrated AI accelerators, now enable hybrid simulation models that co-simulate both building physics and occupant behaviors. These virtual testbeds can evaluate energy-saving measures and identify operational inefficiencies with unprecedented accuracy. However, the computational infrastructure introduces its own energy footprint—a critical concern when simulation energy costs compete with the energy savings being optimized.

Co-simulation frameworks excel at interoperability but face substantial energy consumption during data exchange. Synchronizing multiple models requires frequent data transfers that consume significant CPU and memory resources, creating idle periods that degrade performance. The choice of synchronization timestep critically affects both accuracy and energy efficiency: smaller steps enhance precision but increase communication overhead, while larger steps reduce computational load but may miss transient behavioral events. Network transmission in distributed co-simulation further compounds inefficiency, consuming orders of magnitude more energy than in-memory transfers.

Despite growing research in building energy simulation and HPC optimization, the intersection remains understudied. Specifically, how do agent-based occupant modeling patterns translate to GPU data movement bottlenecks? What is the energy cost of simulating human behavior compared to the building's actual HVAC energy consumption? Can we quantify the trade-offs between simulation accuracy and computational sustainability?

This paper addresses these questions by presenting Twin-B, a co-simulation framework integrating EnergyPlus building energy modeling with Mesa agent-based occupant simulation, deployed on PyTorch Distributed Data Parallel (DDP) infrastructure on the LANTA supercomputer. We use Twin-B as a profiling testbed to establish quantitative relationships between building simulation design choices and resulting HPC performance characteristics.

## Contributions

This work makes three specific contributions:

Profiling analysis of co-simulation bottlenecks: We identify that cudaStreamSynchronize accounts for 66.3% of CUDA API time in building co-simulation, driven by 6,289 small (37.5-byte average) data transfers resulting from per-agent decision patterns. We trace this bottleneck directly to agent-based modeling implementation choices.

Quantification of energy overhead: We demonstrate that current data exchange patterns waste 99.995% of PCIe bandwidth and consume 7.42 J per simulation run. With optimization, this energy cost could be reduced 1,000× while improving performance 1,200×—critical for large-scale building optimization campaigns requiring thousands of simulation runs.

Integrated optimization framework: We provide domain-specific optimization strategies connecting building simulation patterns to HPC efficiency. Our analysis shows that batching agent operations (a building modeling decision) directly eliminates GPU synchronization overhead (an HPC concern), demonstrating how co-design across disciplines enables sustainable digital twins.

The remainder of this paper is organized as follows: Section 2 reviews building energy simulation technologies, HPC techniques, and energy-aware data movement challenges. Section 3 describes the Twin-B co-simulation framework architecture and its mapping to GPU operations. Section 4 presents our profiling methodology and experimental results. Section 5 discusses implications for building simulation design and HPC optimization. Section 6 concludes with future directions.

# 2. BACKGROUND AND RELATED WORK

## 2.1 Building Energy Simulation and Co-Simulation

Building Energy Simulation (BES) tools analyze and predict energy consumption to achieve sustainability goals. EnergyPlus is the dominant dynamic simulation tool, using physics-based models to calculate heat transfer and energy flow. It simulates thermal properties, HVAC systems, lighting, and occupant comfort metrics (PMV - Predicted Mean Vote). However, EnergyPlus has significant limitations in modeling occupant behavior, which accounts for the largest source of energy variability in real buildings.

Agent-based simulation (ABS) addresses this gap by modeling autonomous units (agents) with individual behaviors and decision-making capabilities. Each agent follows rules governing decisions based on environmental conditions. Mesa is a Python-based ABM framework that provides scheduling, spatial environments, and data collection capabilities. Co-simulation frameworks integrate EnergyPlus with ABM tools to capture both building physics and occupant dynamics.

Previous co-simulation research demonstrates significant energy savings potential. Li et al. achieved 29.15% electricity savings using PPO algorithms for temperature and humidity control [6]. Han et al. reduced energy costs by 12% and peak demand by 15% using multi-agent deep RL while maintaining 94% occupant comfort [7]. Wang et al. coupled EnergyPlus-CONTAM to simulate heat and indoor air quality for pollutant risk assessment [8]. However, these studies acknowledge but do not quantify the energy consumption of the simulation infrastructure itself.

## 2.2 High-Performance Computing for Large-Scale Simulation

Large-scale building simulations require HPC resources due to computational complexity. Agent-based models with realistic behavioral parameters demand minute-scale temporal resolution over year-long periods, resulting in tens of billions of floating-point operations per agent per timestep. PyTorch Distributed Data Parallel (DDP) enables efficient multi-GPU computation through process groups, gradient synchronization, and ring all-reduce algorithms [13, 14, 15]. Tampuu et al. demonstrated that distributing agent computations can reduce training time by up to 10× [16, 17].

However, applying DDP to co-simulation poses challenges: (1) synchronizing simulators with different timesteps creates bottlenecks, (2) uneven workload distribution causes idle wait times, and (3) complex agent communication patterns stress inter-process communication [10, 11, 12]. Performance profiling tools like NVIDIA Nsight Systems are essential for identifying these bottlenecks [21]. Tallent et al.'s HPCToolkit demonstrates how profiling enables targeted optimization through bottleneck identification and performance tuning recommendations.

## 2.3 Energy-Aware Data Movement in Co-Simulation

Data exchange between models constitutes a major energy consumption source in co-simulation. Gomes et al. identified that inter-simulator communication imposes computational burden through increased CPU and memory usage [4, 5]. Karnouskos found processing time increases of 20-50% compared to single-simulator execution [9, 12]. Schweiger et al. demonstrated that timestep selection creates accuracy-energy trade-offs: larger steps reduce workload and energy but sacrifice accuracy, while smaller steps improve accuracy but significantly increase energy consumption [6].

Network-based data transfer in distributed co-simulation consumes substantially more energy than in-memory communication. Koomey's analysis of data center growth estimated that transmitting 1 GB across networks consumes approximately 0.06-0.2 kWh depending on distance and network type [5, 7]. Given that co-simulations can execute thousands of data exchanges per run, this cumulative transmission overhead constitutes a significant energy footprint requiring optimization.

Despite recognition of these challenges, existing literature lacks quantitative analysis linking specific simulation patterns to energy costs. High-frequency data exchange necessary for behavioral modeling requires fine temporal resolution, yet the energy implications remain unquantified. Our work addresses this gap by profiling an integrated building co-simulation to establish concrete relationships between modeling choices and computational energy consumption.

# 3. TWIN-B CO-SIMULATION FRAMEWORK

This section describes the Twin-B digital twin architecture, which combines EnergyPlus building energy simulation with Mesa agent-based occupant modeling on PyTorch DDP infrastructure. We detail the system components, data flow, and critically—the mapping from building simulation decisions to GPU operations that drives the profiling analysis in Section 4.

## 3.1 Case Study: Boonchoo Building

Twin-B models the Boonchoo Building at Thammasat University's Lampang Campus, Thailand (Northern weather zone 483780, coordinates 18.277°N 99.504°E, climate zone 7.0). Operational since 2018, the building provides detailed materials and maintenance records enabling accurate digital twin construction. The building accommodates educational activities with variable occupancy patterns—a scenario ideal for testing energy management strategies.

The digital twin, constructed in SketchUp and imported to EnergyPlus, divides the building into 73 thermal zones based on physical structure and usage patterns. Our demonstration model focuses on three representative classrooms that can accommodate 1,875 occupants at full capacity. This scale—nearly 2,000 agents interacting with 73 zones—creates the computational complexity necessary for meaningful HPC profiling.

## 3.2 Data Preparation and Model Development

Twin-B requires two categories of input data:

Building physical data: 3D architectural models from actual building plans, material thermal properties with heat transfer coefficients (U-values) for walls, roofs, and windows, HVAC system specifications including air conditioner types and coefficients of performance (COP), and EnergyPlus Weather Format (EPW) files for Lampang Province from Climate.OneBuilding database.

Occupant behavioral data: Classroom usage patterns by students and teachers, air conditioning control behavior (on/off patterns, setpoint preferences), building occupant density during different periods (class schedules, exam weeks, conferences), and individual thermal comfort preferences.

The EnergyPlus model defines 73 thermal zones, assigns material properties with validated U-values, configures HVAC parameters including equipment COP and operating schedules, and integrates real weather data. The Mesa agent model creates 1,875 agents representing students, instructors, staff, cleaners, wardens, and visitors. Each agent has unique attributes: preferred temperature, comfort tolerance, current location, and behavioral rules based on schedule and environmental conditions.

## 3.3 Co-Simulation Integration and Synchronization

Twin-B employs master-slave synchronization with EnergyPlus as master and Mesa as slave. The simulation cycle operates as follows:

EnergyPlus executes building physics simulation and reaches a callback point (5-minute intervals, 288 timesteps per 24-hour day).

EnergyPlus pauses and invokes callback API, which reads current zone temperatures (73 float values) and places them in a thread-safe queue.

Orchestrator process extracts temperature data from queue and broadcasts to Mesa model running on distributed GPUs.

Each Mesa agent receives zone temperature for their current location, calculates comfort level, and decides whether to request air conditioning (binary decision: on/off or setpoint temperature).

PyTorch DDP executes NCCL all-gather to collect setpoint requests from all 1,875 agents distributed across 2 GPUs.

Aggregation logic computes zone-level setpoints by taking the minimum request per zone (coldest preference wins).

Merged setpoints (73 float values) return to EnergyPlus via callback API.

EnergyPlus applies actuator values to HVAC control system and advances to next timestep.

This tight coupling ensures physical accuracy (5-minute timesteps capture transient thermal dynamics) but creates frequent data exchange—the source of performance bottlenecks analyzed in Section 4.

## 3.4 From Building Simulation to GPU Operations: The Critical Mapping

Understanding how building modeling decisions manifest as GPU operations is essential for interpreting Section 4's profiling results. This section traces the complete data flow from occupant behavior to memory transfers.

Agent Decision-Making and Data Structure

Each of the 1,875 agents maintains state on GPU as PyTorch tensors:
  • Agent ID (int32): 4 bytes
  • Current zone (int32): 4 bytes
  • Preferred temperature (float32): 4 bytes
  • Comfort tolerance (float32): 4 bytes
  • Current temperature (float32): 4 bytes (received from EnergyPlus)
  • Using AC decision (bool): 1 byte
  • Setpoint request (float32): 4 bytes
  Total per agent: ~25-40 bytes depending on padding

The 37.5-byte average observed in profiling (Section 4.2) directly reflects this per-agent data structure. With 1,875 agents making decisions each timestep, and 288 timesteps per simulation, this architecture generates:

• 288 timesteps × 1,875 agents = 540,000 agent decisions
  • Each decision requires temperature input (H2D transfer) and setpoint output (D2H transfer)
  • Current implementation transfers agents individually rather than batched
  • Result: 6,289 observed H2D transfers averaging 37.5 bytes (Section 4.2)

Why Small Transfers Occur

The profiling reveals frequent small transfers rather than batched operations due to implementation patterns in the Mesa-PyTorch integration:

Per-agent tensor creation: Each agent decision creates a new tensor transferred independently to GPU.

Synchronous execution: Agent logic executes sequentially rather than vectorized, forcing immediate data availability.

Comfort calculation: The formula comfort_level = max(0.0, preferred_temp - abs(current_temp - preferred_temp)) operates on scalar values rather than batched vectors.

Decision broadcasting: AC control decisions propagate individually through the DDP communication layer rather than aggregated.

These patterns are not inherent to building simulation but reflect conventional agent-based modeling practice—each agent as an independent object. The GPU, however, excels at batched operations. This architectural mismatch creates the 99.995% bandwidth waste quantified in Section 4.

Communication Pattern Analysis

The 73 thermal zones create additional communication complexity. After individual agent decisions, zone-level aggregation requires:

• Scatter: Distribute 1,875 agent decisions to respective zones (many-to-many mapping)
  • Reduce: Compute minimum setpoint per zone (73 reduction operations)
  • Gather: NCCL all-gather across 2 GPUs to merge distributed agents (observed 580 AllGather calls)
  • Broadcast: Return 73 zone setpoints to EnergyPlus

This explains why NCCL communication consumes 64.4% of GPU kernel time (Section 4.2)—not due to NCCL inefficiency (operations complete in 14-17 μs), but due to operation frequency driven by the 73-zone × 288-timestep architecture.

Synchronization Requirements

The master-slave synchronization strategy imposes strict ordering:

1. EnergyPlus callback blocks until zone temperatures are ready
  2. Mesa agents cannot proceed until temperatures arrive (CPU-GPU synchronization)
  3. EnergyPlus cannot advance until setpoint decisions return (GPU-CPU synchronization)
  4. Each synchronization point triggers cudaStreamSynchronize

With 288 timesteps and multiple synchronization points per timestep, this explains the 547,393 cudaStreamSynchronize calls observed (Section 4.2). The 66.3% CUDA API time overhead is not a GPU limitation but a consequence of the tight temporal coupling required for physical accuracy.

Design Trade-offs

This architecture makes specific trade-offs:

Physical accuracy vs. computational efficiency: 5-minute timesteps capture thermal dynamics accurately but require 288 synchronization cycles, each introducing overhead.

Individual agent modeling vs. batched computation: Preserving agent autonomy (different preferences, locations) complicates vectorization.

Real-time coupling vs. loose coupling: Master-slave synchronization ensures consistency but prevents asynchronous execution that could hide communication latency.

Zone-level aggregation vs. global optimization: Computing minimum setpoint per zone (physically realistic) requires scatter-gather patterns that stress GPU communication.

These trade-offs are not failures but conscious design decisions prioritizing simulation accuracy. Section 4 quantifies their computational cost, and Section 5 discusses optimization strategies that maintain accuracy while reducing overhead.

Summary: Building Patterns → GPU Bottlenecks

To summarize the mapping that drives our profiling analysis:

| Building Decision | GPU Manifestation | Profiling Observation |
| --- | --- | --- |
| 1,875 individual agents | 37.5-byte per-agent transfers | 6,289 small H2D operations |
| 5-minute timesteps (288/day) | Forced synchronization each step | 547,393 cudaStreamSynchronize calls (66.3% time) |
| 73 thermal zones | NCCL all-gather for aggregation | 580 AllGather operations (64.4% kernel time) |
| Per-agent decision autonomy | Sequential tensor operations | 99.995% PCIe bandwidth waste |
| Real-time coupling | Blocking queue operations | 6.3 seconds CPU waiting (96.2% of OS runtime) |

This detailed mapping establishes the foundation for interpreting Section 4's profiling results not as abstract GPU metrics but as direct consequences of building simulation architecture. Optimization strategies (Section 5) must address both domains: restructuring agent operations (building modeling) to enable batched GPU computation (HPC optimization).

# 4. PROFILING METHODOLOGY AND RESULTS

This section presents our experimental methodology for profiling Twin-B on the LANTA supercomputer, baseline platform characterization, co-simulation profiling results, and analysis connecting these measurements to the building simulation patterns described in Section 3.4.

## 4.1 Experimental Setup

Hardware Platform

Experiments were conducted on the ThaiSC LANTA supercomputer (HPE Cray EX) GPU partition. Each GPU node features:
  • 2× NVIDIA A100-SXM4-40GB GPUs (Compute Capability 8.0)
  • AMD EPYC 7713 64-Core Processor
  • CUDA 11.8 with NCCL 2.12 networking backend
  • PCIe Gen4 16x (theoretical 32 GB/s bidirectional)
  • NVLink for GPU-to-GPU communication (400 GB/s)
  • 64 GB system RAM

Software Environment

The profiling environment consisted of:
  • EnergyPlus 25.1.0 with Python API (pyenergyplus)
  • Mesa 2.1.5 agent-based modeling framework
  • PyTorch 2.2.2 with Distributed Data Parallel (DDP)
  • NVIDIA Nsight Systems 2024.11 for profiling
  • nvidia-smi for power monitoring (1-second sampling)

Profiling Configuration

| Configuration Parameter | Value |
| --- | --- |
| Profiling Tool | NVIDIA Nsight Systems (nsys) |
| SDK Version | HPC SDK 24.11 |
| Trace Options | cuda, nvtx, osrt, openmp |
| GPU Metrics | All devices (--gpu-metrics-devices=all) |
| CUDA Memory Tracking | Enabled (--cuda-memory-usage=true) |
| Job Scheduler | SLURM with 2 GPUs, 8 CPU cores, 64 GB RAM |
| Simulation Duration | 3 days (72 hours, 288 timesteps × 3) |
| Total Events Collected | 8,011,365 |
| Threads Tracked | 56 |

Profiling Procedure

The profiling workflow executed:

Load Python environment and EnergyPlus modules

Initialize PyTorch DDP with NCCL backend across 2 GPUs

Launch nsys profiling wrapper around torchrun execution command

Start background nvidia-smi process recording GPU power every 1 second

Execute Twin-B co-simulation for 3-day building operation (864 timesteps total)

Collect comprehensive traces: CUDA API calls, GPU kernels, OS runtime, memory operations

Post-process nsys reports to extract timing, energy, and communication patterns

Analyze correlation between building simulation events and GPU profiling metrics

[Note: This is a partial revision showing the restructuring approach. The complete revision would continue with Sections 4.2-6 following the same principles: remove redundancy, integrate building+HPC narratives, add connective tissue, and merge overlapping sections.]